Ex.No.10.  Design a simple topology and configure with one router, two switches and PCs using Cisco Packet Tracer

AIM:

To design and configure a simple network topology with one router, two switches,

and four PCs in Cisco Packet Tracer for successful inter-network communication.

PROCEDURE:

1. Topology

Router R1

┌────────────┐

│ │

G0/0 G0/1

│ │

│ │

Switch S1 Switch S2

┌────┴────┐ ┌────┴────┐

│ │ │ │

PC1 PC2 PC3 PC4

2. Devices to place

In Cisco Packet Tracer, place:

 1 Router – Cisco 1941

 2 Switches – Cisco 2960

 4 PCs

Use Copper Straight-Through cables.

3. IP Addressing

Device Interface IP Address Subnet Mask Gateway

R1 G0/0 192.168.1.1 255.255.255.0 —

R1 G0/1 192.168.2.1 255.255.255.0 —

PC1 NIC 192.168.1.10 255.255.255.0 192.168.1.1

PC2 NIC 192.168.1.11 255.255.255.0 192.168.1.1

PC3 NIC 192.168.2.10 255.255.255.0 192.168.2.1

PC4 NIC 192.168.2.11 255.255.255.0 192.168.2.1

So we have two different networks:

Network 1: 192.168.1.0/24

PC1 ─┐

PC2 ─┤

S1 ─── R1 G0/0 = 192.168.1.1

Network 2: 192.168.2.0/24

PC3 ─┐

PC4 ─┤

S2 ─── R1 G0/1 = 192.168.2.1

4. Configure Router R1

Click R1 → CLI and enter:

enable

configure terminal

interface gigabitEthernet 0/0

ip address 192.168.1.1 255.255.255.0

no shutdown

exit

interface gigabitEthernet 0/1

ip address 192.168.2.1 255.255.255.0

no shutdown

exit

end

write memory

The important command is:

no shutdown

It activates the router interface.

Check the interfaces using:

show ip interface brief

You should see the interfaces as up/up.

OUTPUT:

RESULT:

Thus configuration with one router, two switches and PC’s using cisco packet tracer is designed