Ex.No.12. Examine Network Address Translation (NAT) using Cisco Packet Tracer

AIM

To configure and examine Network Address Translation (NAT) using Cisco Packet Tracer

and verify communication between a private network and a public network.

Requirements

1.Cisco Packet Tracer

1 Router (Cisco 1941)

2 Switches (Cisco 2960)

2 PCs

Copper Straight-Through cables

Network Topology

Private Network Public Network

PC0 -------- Switch0 -------- Router -------- Switch1 -------- PC1

192.168.1.10 NAT Router 203.0.113.10

G0/0 Public PC

192.168.1.1

|

G0/1

203.0.113.1

IP Addressing

Device Interface IP Address Subnet Mask

PC0 FastEthernet0 192.168.1.10 255.255.255.0

Router G0/0 192.168.1.1 255.255.255.0

Router G0/1 203.0.113.1 255.255.255.0

PC1 FastEthernet0 203.0.113.10 255.255.255.0

PROCEDURE

Step 1: Create the topology

1. Open Cisco Packet Tracer.

2. Place:

o Two PCs.

o Two 2960 switches.

o One 1941 router.

3. Connect:

o PC0 → Switch0.

o Switch0 → Router G0/0.

o Router G0/1 → Switch1.

o Switch1 → PC1.

4. Use Copper Straight-Through cables.

Step 2: Configure PC0

Go to:

PC0 → Desktop → IP Configuration

Enter:

IP Address : 192.168.1.10

Subnet Mask : 255.255.255.0

Default Gateway : 192.168.1.1

Step 3: Configure PC1

Go to:

PC1 → Desktop → IP Configuration

Enter:

IP Address : 203.0.113.10

Subnet Mask : 255.255.255.0

Default Gateway : 203.0.113.1

Step 4: Configure the Router

Open:

Router → CLI

Configure the inside interface:

Router&gt; enable

Router# configure terminal

Router(config)# interface gigabitEthernet 0/0

Router(config-if)# ip address 192.168.1.1 255.255.255.0

Router(config-if)# ip nat inside

Router(config-if)# no shutdown

Router(config-if)# exit

Configure the outside interface:

Router(config)# interface gigabitEthernet 0/1

Router(config-if)# ip address 203.0.113.1 255.255.255.0

Router(config-if)# ip nat outside

Router(config-if)# no shutdown

Router(config-if)# exit

Step 5: Configure NAT

Create an access list to identify the private network:

Router(config)# access-list 1 permit 192.168.1.0 0.0.0.255

Configure PAT (NAT Overload):

Router(config)# ip nat inside source list 1 interface gigabitEthernet 0/1 overload

This allows multiple private IP addresses to share the router&#39;s outside IP address.

Step 6: Verify the Router Configuration

Execute:

Router# show ip interface brief

Expected output:

Interface IP-Address Status

GigabitEthernet0/0 192.168.1.1 up

GigabitEthernet0/1 203.0.113.1 up

Step 7: Test Connectivity

From PC0, open:

Desktop → Command Prompt

Enter:

ping 192.168.1.1

Expected:

Reply from 192.168.1.1: bytes=32 time&lt;1ms TTL=255

Now test the public-side PC:

ping 203.0.113.10

Expected:

Reply from 203.0.113.10: bytes=32 time&lt;1ms TTL=127

Step 8: Verify NAT Translation

Generate traffic from PC0 by pinging PC1:

PC0&gt; ping 203.0.113.10

Then, on the router, enter:

Router# show ip nat translations

Sample output:

Pro Inside global Inside local Outside local Outside global

icmp 203.0.113.1:1 192.168.1.10:1 203.0.113.10:1 203.0.113.10:1

This shows that the private IP address 192.168.1.10 has been translated to the router&#39;s public

IP address 203.0.113.1.

Step 9: Verify NAT Statistics

Enter:

Router# show ip nat statistics

Sample output:

Total active translations: 1

Inside interfaces:

GigabitEthernet0/0

Outside interfaces:

GigabitEthernet0/1

Hits: 5

Misses: 0

The exact numbers may vary depending on the number of packets generated.

SAMPLE OUTPUT

PC0 pinging PC1:

PC&gt; ping 203.0.113.10

Pinging 203.0.113.10 with 32 bytes of data:

Reply from 203.0.113.10: bytes=32 time&lt;1ms TTL=127

Reply from 203.0.113.10: bytes=32 time&lt;1ms TTL=127

Reply from 203.0.113.10: bytes=32 time&lt;1ms TTL=127

Reply from 203.0.113.10: bytes=32 time&lt;1ms TTL=127

Ping statistics:

Packets: Sent = 4, Received = 4, Lost = 0 (0% loss)

NAT Translation Table:

Router# show ip nat translations

Pro Inside global Inside local Outside local Outside global

icmp 203.0.113.1:1 192.168.1.10:1 203.0.113.10:1 203.0.113.10:1

OUTPUT:

RESULT

Thus, Network Address Translation (NAT) was successfully configured and examined using

Cisco Packet Tracer. The private IP address 192.168.1.10 was translated into the public IP

address 203.0.113.1, and successful communication with the public network was verified.